Initial Setup for Romer Arms

- Download the Romer Europe dll install zip, RomerEurope.zip, from ftp://ftp.kinematics.com/pub/SA/Install/Driver Downloads/RomerEuropeArms

Then, unzip into your current SA install folder.  (See NOTES section below.)

- Run the arm data install from the floppy disk included with your arm.  This will do the following:

1) Creates a directory, C:\ARM\Armdata.  In this directory, the arm configuration file, armspecs.gds, will be written.

2) Creates a directory, C:\ARM\Probes.  In this directory, the probe definitions, probe1.gds, etc., will be written.

3) Writes information regarding the arm to the registry.

Now, you may need to run Romer’s Romosoft application to configure the arm.  If you cannot connect at this point, follow the instructions in ‘Romer EU Arm Configuration with Romosoft.doc’.

You can now add a Romer Sigma Arm to your SA job, and begin using the arm:

Arm Settings

- To edit arm ip address, volume, etc., hit the Arm Settings (Check Mark, lower left) button, and then hit the 'Arm Settings' button in the Options dialog.  You’ll then see the GDS settings dialog:

Notes

Your SA install directory must contain the following files (from RomerEurope.zip):

fctlmain.dll

GDSDLL.dll

mil.dll

milblob.dll

milcal.dll

milim.dll

rxxc.dll

sdrv32.dll

sx32w.dll

tck32.dll

trackerapi.dll

You may recognize 3 of the last 4 as being Automated Precision, Inc. dll's.

This is true, but these are NOT current API dlls.

This will render API trackers un-runnable, since SA supports API through current API dll's.

Unless or until Romer Europe decides to remove these dll's from their sdk, you may solve this by simply renaming your SA folder from which you run the Romer arm, and re-install another copy of SA to the default folder, from which you can still run API trackers.