Romer Arm Configuration Procedure

1) Download the Romer Europe dll install zip, RomerEurope.zip, from ftp://ftp.kinematics.com/pub/SA/Install/Driver Downloads/RomerEuropeArms

Then, unzip into your current SA install folder. (See RomerEurope\Readme.txt in your SA install folder for more information.)

2) Run the arm data install from the floppy disk included with your arm.  This will do the following:

a) Creates a directory, C:\ARM\Armdata.  In this directory, the arm configuration file, armspecs.gds, will be written.

b) Creates a directory, C:\ARM\Probes.  In this directory, the probe definitions, probe1.gds, etc., will be written.

c) Writes information regarding the arm to the registry.

Now follow these instructions from Romer….

3°) Run Romosoft, then right click on GDS icon and select “board”

Check that “Ethernet board” has been validated as shown below:

4°) Run Romosoft, then select GDS, and GDS Function

The Arm’s black hub box must be ON

Select “Change Arm” and in the new screen (see below), select the two points “..” and activate.

Now, the software will be able to use the arm data inside the correct directory : C:\ARM, as shown below:

5°) In the following screen, click “CTRL”+ “Shift” + “o”

Then select “GDS dialog box”, and check the “adresse IP et masque” (=TCP/IP address and mask)

In this box, you must enter the IP address of your arm

Click Ok

6°) Then, run 3rd party software (SA Interface)

Changing the IP Address

If you need to change the ip address, just go to the “GDS Dialog Box” as described above, and select the Sigma setup tab.  First, be sure the current ip is entered, and confirm by pressing ‘Read’.  You should get a ‘Successful operation’ message.  Then, enter the desired ip, and press ‘Write’.  Once this is confirmed, go back to the ‘General’ tab, and be sure the new ip is shown there.  Then toggle power on the control box, close the GDS app, and re-run once more to confirm the new ip.