MP String commands for setting CMM Arm Measure Modes…

Can be used with Instrument Operations ‘Configure and Measure’ or ‘Set Instrument Measurement Mode/Profile’

(Hover over arm measure buttons – tool tip shows appropriate string)

| String Command | Description |
| --- | --- |
| "Discrete" | Single Discrete Point |
| "Stream" | Scan Points per User Option Setting (spatial or temporal increment) |
| "Patch" | Measure Patch, or projected point |
| "Pin" | Measure Pin, or outside circle with projection plane |
| "Slot" | Measure Slot, or two inside circles with projection plane |
| "Line" | Measure Line per User Option Setting (Two point, averaged, or edge) |
| "Plane" | Measure Plane |
| "Sphere" | Measure Sphere |
| "Section" | Measure Cross Sections (multiple if cross value not equal to zero) |
| "Frame" | Measure Frames (origin at probe center, using probe orientation) |
| "Batch" | Perform Guided Measurement (invoked from SA with Batch of pts.) |
| "Scanner" | If available, use installed line scanner to measure cloud points |
| "Average" | Single Averaged Point |